import numpy as np # --- 1. Constants & Aircraft Parameters (Cessna 172) --- m = 900.0 # Mass (kg) g = 9.81 # Gravity (m/s^2) S = 16.2 # Wing Area (m^2) b = 11.0 # Wingspan (m) rho = 1.225 # Air density (kg/m^3) CD0 = 0.08 # Parasite Drag (Full Flaps) K_inf = 0.055 # Induced Drag Factor (Out of ground effect) CL_alpha = 4.4 # Lift Curve Slope (per rad) T_initial = 250.0 # Initial Thrust (N) T_idle = -50.0 # Idle Thrust / Disc Drag (N) # --- 2. Simulation Setup --- dt = 0.01 # Time step (s) h_start = 15.24 # 50 ft in meters V_start = 37.56 # 73 KIAS in m/s gamma_start = np.radians(-4.17) alpha_start = np.radians(1.0) # Initial pitch for approach # State Vector: [Velocity, Flight Path Angle, Altitude, Distance, Alpha] y = np.array([V_start, gamma_start, h_start, 0.0, alpha_start]) def get_derivatives(t, y): V, gamma, h, x, alpha = y # --- A. Control Logic (Scheduling) --- # 1. Thrust Decay (shuttles from 250N to idle over 2 seconds) thrust = max(T_idle, T_initial - 150.0 * t) # 2. Alpha Controller (Flare Logic) # Flare starts at ~6 meters (20 ft) if h < 6.0: # Target sink rate (h_dot) = -h / tau (Tau = 3.0s) # We want to touch down at approx -0.3 m/s h_dot_target = -max(h / 3.0, 0.3) h_dot_actual = V * np.sin(gamma) # Proportional control to change alpha rate alpha_dot = 0.4 * (h_dot_target - h_dot_actual) # Limit alpha rate to 3 deg/sec and max alpha to 8 deg (0.139 rad) if alpha >= np.radians(8.0) and alpha_dot > 0: alpha_dot = 0 else: alpha_dot = 0 # Hold approach alpha until 20ft # --- B. Aerodynamics --- # Ground Effect factor (Cheung-Wiesel) phi = ((16*h/b)**2) / (1 + (16*h/b)**2) K_eff = K_inf * phi CL = CL_alpha * alpha CD = CD0 + K_eff * (CL**2) L = 0.5 * rho * (V**2) * S * CL D = 0.5 * rho * (V**2) * S * CD # --- C. Equations of Motion --- dV_dt = (thrust - D) / m - g * np.sin(gamma) dGamma_dt = (L - m * g * np.cos(gamma)) / (m * V) dh_dt = V * np.sin(gamma) dx_dt = V * np.cos(gamma) return np.array([dV_dt, dGamma_dt, dh_dt, dx_dt, alpha_dot]) # --- 3. RK4 Integration Loop --- results = [] t = 0 while y[2] > 0: # Stop when altitude <= 0 k1 = get_derivatives(t, y) k2 = get_derivatives(t + dt/2, y + dt/2 * k1) k3 = get_derivatives(t + dt/2, y + dt/2 * k2) k4 = get_derivatives(t + dt, y + dt * k3) y = y + (dt/6.0) * (k1 + 2*k2 + 2*k3 + k4) t += dt results.append([t, *y]) # --- 4. Final Output --- res = np.array(results)[-1] print(f"Touchdown Time: {res[0]:.2f} s") print(f"Touchdown Speed: {res[1] * 1.94384:.2f} KIAS") print(f"Touchdown Sink Rate: {res[1] * np.sin(res[2]) * 196.85:.2f} fpm") print(f"Final Alpha: {np.degrees(res[5]):.2f} deg")